| Existing & Proposed Sindh Environmental Quality Standards for Ambient Air Quality |  |  |  |
| --- | --- | --- | --- |
| S. No. | Parameter | Existing SEPA Standards | Proposed SEQS (Sindh EPA) |
| 01 | Sulphur Dioxide (SO2) | Annual= 80ug/m3 24hrs= 120ug/m3 | Annual= 50ug/m3 24hrs= 80ug/m3 |
| 02 | NO | Annual= 40ug/m3 24hrs= 40ug/m3 | Annual= 40ug/m3 24hrs= 40ug/m3 |
| 03 | NO2 | Annual= 40ug/m3 24hrs= 80ug/m3 | Annual= 40ug/m3 24hrs= 80ug/m3 |
| 04 | O3 | 1 hr= 130ug/m3 | 1 hr = 130ug/m3 |
| 05 | Suspended Particulate Matters (SPM) | Annual= 360ug/m3 24hrs= 500ug/m3 | Annual= 360ug/m3 24hrs= 500ug/m3 |
| 06 | Lead (Pb) | Annual= 1ug/m3 24hrs= 1.5 ug/m3 | Annual= 0.5 ug/m3 24hrs = 1ug/m3 |
| 07 | Carbon | 8 hr= 5mg/m3 | 8 hr = 5mg/m3 |
| 08 | Carbon Monoxide (CO) | 1 hr= 10mg/m3 | 1 hr = 10mg/m3 |
| 09 | PM10 | Annual= 120ug/m3 24hrs= 150ug/m3 | Annual= 60 ug/m3 24hrs= 100ug/m3 |
| 10 | PM2.5 | Annual= 40 ug/m3 24hrs= 75 ug/m3 | Annual= 15 ug/m3 24hrs = 35 ug/m3 1hrs = 15 ug/m3 |

| Existing & Proposed Sindh Environmental Quality Standards for Industrial Gaseous Emission (mg/Nm3) |  |  |  |
| --- | --- | --- | --- |
| S. No. | Parameter | Existing SEPA Standards | Proposed SEQS (Sindh EPA) |
| 01 | Smoke | 40% or 2 Ringleman Scale or equivalent smoke number | 40% or 2 Ringleman Scale or equivalent smoke number |
| 02 | PM | 100 | 10-120 mg/Nm3 |
| 03 | Sulphur Oxides | 1700 mg/Nm3 | 200-400 mg/Nm3 (Coal-fired power plant ) 10-20 mg/Nm3 (Gas-fired power plant) 300-500 mg/Nm3(Lignite-fired power plant ) |
| 04 | Carbon Monoxide (CO) | 800 mg/Nm3 | 100-200 mg/Nm3 |
| 05 | Lead | 50 mg/Nm3 | 0.5-1 mg/Nm3 |
| 06 | Mercury | 10 mg/Nm3 | 0.03-0.1 mg/Nm3 |
| 07 | Cadmium | 20 mg/Nm3 | 0.1-0.2 mg/Nm3 |
| 08 | Arsenic | 20 mg/Nm3 | 0.1-0.2 mg/Nm3 |
| 09 | Copper | 50 mg/Nm3 | 0.5-1 mg/Nm3 |
| 10 | Antimony | 20 mg/Nm3 | 20 mg/Nm3 |
| 11 | Zinc | 200 mg/Nm3 | 1-2 mg/Nm3 |
| 12 | Oxides of Nitrogen | 1200 mg/Nm3 (Coal-fired power plant ) 400 mg/Nm3 (Gas-fired power plant) 600 mg/Nm3(Oil-fired power plant ) | 300-600 mg/Nm3 (Coal-fired power plant ) 50-100 mg/Nm3 (Gas-fired power plant) 400-700 mg/Nm3(Lignite-fired power plant ) |

| Existing & Proposed Sindh Environmental Quality Standards for Municipal and Liquid Industrial Effluents (mg/L) |  |  |  |
| --- | --- | --- | --- |
| S. No. | Parameter | Existing SEPA Standards | Proposed SEQS (Sindh EPA) |
| 01 | Biological Oxygen Demand (BOD) at 20˚C | 80 | 60 |
| 02 | Chemical Oxygen Demand (COD) | 150 | 150 |
| 03 | Total Suspended Solids (TSS) | 200 | 150 |
| 04 | Total Dissolved Solids (TDS) | 3500 | 2100 |
| 05 | Temperature 40˚ or Temperature Increase | <3˚C | <3˚C |
| 06 | pH Value | 6 - 9 | 6 - 9 |
| 07 | Oil and Grease | 10 | 10 |
| 08 | Phenolic Compounds (as phenol) | 0.1 | 1 |
| 09 | Chloride (as CL-) | 1000 | 1000 |
| 10 | Fluoride (as F-) | 10 | 10 |
| 11 | Cyanide (as CN-) total | 1 | 0.2 |
| 12 | An-ionic detergents (as MBAS) (2) | 20 | 20 |
| 13 | Sulphate (SO₄²¯) | 600 | 600 |
| 14 | Sulphide (S²¯) | 1 | 1 |
| 15 | Ammonia (NH₃) | 40 | 40 |
| 16 | Pesticides | 0.15 | 0.15 |
| 17 | Cadmium | 0.1 | 0.1 |
| 18 | Chromium (trivalent and hexavalent) | 1 | 0.5 |
| 19 | Copper | 1 | 1 |
| 20 | Lead | 0.5 | 0.1 |
| 21 | Mercury | 0.01 | 0.01 |
| 22 | Salenium | 0.5 | 0.05 |
| 23 | Nickle | 1 | 1 |
| 24 | Silver | 1 | 0.5 |
| 25 | Total Toxic Metals | 2 | Undesirable |
| 26 | Zinc | 5 | 5 |
| 27 | Arsenic | 1 | 0.5 |
| 28 | Barium | 1.5 | 1.5 |
| 29 | Iron | 8 | 3.0 |
| 30 | Manganese | 1.5 | 1.5 |
| 31 | Boron | 6 | 2.0 |
| 32 | Chlorine | 1 | 1 |